# C11.xl2

Flexible end plate welded to to sec. beam, bolted to primary (or plate on it)

# General

This connection is considered as a pin type connection. Small moments and axial actions might be input.

Standard: Eurocode

Bolt eccentr. calc. method: Elastic

Connection axis: Main beam - plate contact point

Connection type: Coincident Top of Steel

Category for shear: Bearing type

Plate welding to secondary: Web and flanges

Moment calc. method: Eurocode/BS #42

Sec. plate bolted to: Plate welded to flanges

Seismicity: Joint is NOT dissipative

# Loads

Units: kN, kN*m

Case= 1, N Ed= -1.24, Vmaj Ed= -3.27, Mmaj Ed= 4.69, case name= ULS_SC_Comp,

Case= 2, N Ed= 0.55, Vmaj Ed= 2.91, Mmaj Ed= -4.67, case name= ULS_SC_Tens,

Case= 3, N Ed= -1.68, Vmaj Ed= -3.26, Mmaj Ed= 4.71, case name= ULS_SC_Vx+_Comp,

Case= 4, N Ed= 0.5, Vmaj Ed= 2.92, Mmaj Ed= -4.7, case name= ULS_SC_Vx+_Tens,

Case= 5, N Ed= -2.16, Vmaj Ed= -2.96, Mmaj Ed= 4.29, case name= ULS_Vx+_SC_Comp,

Case= 6, N Ed= 0.46, Vmaj Ed= 2.68, Mmaj Ed= -4.28, case name= ULS_Vx+_SC_Tens,

Case= 7, N Ed= -1.12, Vmaj Ed= 1.39, Mmaj Ed= -1.98, case name= ULS_Vx+_Comp,

Case= 8, N Ed= 1.37, Vmaj Ed= 0.95, Mmaj Ed= -1.98, case name= ULS_Vx+_Tens,

Case= 9, N Ed= -2.06, Vmaj Ed= -3.28, Mmaj Ed= 4.67, case name= ULS_SC_Vx-_Comp,

Case= 10, N Ed= 0.61, Vmaj Ed= 2.9, Mmaj Ed= -4.66, case name= ULS_SC_Vx-_Tens,

Case= 11, N Ed= -2.55, Vmaj Ed= -2.99, Mmaj Ed= 4.22, case name= ULS_Vx-_SC_Comp,

Case= 12, N Ed= 0.77, Vmaj Ed= 2.64, Mmaj Ed= -4.27, case name= ULS_Vx-_SC_Tens,

Case= 13, N Ed= -1.56, Vmaj Ed= -1.05, Mmaj Ed= 2.03, case name= ULS_Vx-_Comp,

Case= 14, N Ed= 1.37, Vmaj Ed= 1.36, Mmaj Ed= -2.03, case name= ULS_Vx-_Tens,

Case= 15, N Ed= -10.48, Vmaj Ed= -3.06, Mmaj Ed= 4.69, case name= ULS_Vy+_SC_Comp,

Case= 16, N Ed= 0.4, Vmaj Ed= -2.14, Mmaj Ed= 2.76, case name= ULS_Vy+_SC_Tens,

Case= 17, N Ed= -10.33, Vmaj Ed= 1.3, Mmaj Ed= -1.7, case name= ULS_Vy+_Comp,

Case= 18, N Ed= 1.11, Vmaj Ed= 0.98, Mmaj Ed= -1.24, case name= ULS_Vy+_Tens,

Case= 19, N Ed= -6.71, Vmaj Ed= -3.32, Mmaj Ed= 4.95, case name= ULS_SC_Vy+_Comp,

Case= 20, N Ed= 0.29, Vmaj Ed= -2.19, Mmaj Ed= 2.82, case name= ULS_SC_Vy+_Tens,

Case= 21, N Ed= -10.91, Vmaj Ed= -2.98, Mmaj Ed= 4.36, case name= ULS_Vy-_SC_Comp,

Case= 22, N Ed= 0.48, Vmaj Ed= 2.67, Mmaj Ed= -4.27, case name= ULS_Vy-_SC_Tens,

Case= 23, N Ed= -10.24, Vmaj Ed= 1.37, Mmaj Ed= -1.99, case name= ULS_Vy-_Comp,

Case= 24, N Ed= 0.48, Vmaj Ed= 0.56, Mmaj Ed= -1.29, case name= ULS_Vy-_Tens,

Case= 25, N Ed= -6.94, Vmaj Ed= -3.28, Mmaj Ed= 4.71, case name= ULS_SC_Vy-_Comp,

Case= 26, N Ed= 0.53, Vmaj Ed= 2.92, Mmaj Ed= -4.7, case name= ULS_SC_Vy-_Tens,

Case= 27, N Ed= -12.39, Vmaj Ed= -3.28, Mmaj Ed= 4.73, case name= ULS_SC_T+_Comp,

Case= 28, N Ed= 0.56, Vmaj Ed= 2.93, Mmaj Ed= -4.71, case name= ULS_SC_T+_Tens,

Case= 29, N Ed= -19.73, Vmaj Ed= -2.98, Mmaj Ed= 4.32, case name= ULS_T+_SC_Comp,

Case= 30, N Ed= 0.51, Vmaj Ed= 2.69, Mmaj Ed= -4.3, case name= ULS_T+_SC_Tens,

Case= 31, N Ed= -11.99, Vmaj Ed= 1.39, Mmaj Ed= -1.97, case name= ULS_Vx+_T+_Comp,

Case= 32, N Ed= 1.37, Vmaj Ed= 0.89, Mmaj Ed= -1.97, case name= ULS_Vx+_T+_Tens,

Case= 33, N Ed= -0.92, Vmaj Ed= -1.04, Mmaj Ed= 1.99, case name= ULS_Vx+_T-_Comp,

Case= 34, N Ed= 11.9, Vmaj Ed= 1.4, Mmaj Ed= -2, case name= ULS_Vx+_T-_Tens,

Case= 35, N Ed= -12.69, Vmaj Ed= -3.27, Mmaj Ed= 4.75, case name= ULS_SC_Vx+_T+_Comp,

Case= 36, N Ed= 0.5, Vmaj Ed= 2.95, Mmaj Ed= -4.74, case name= ULS_SC_Vx+_T+_Tens,

Case= 37, N Ed= -1.14, Vmaj Ed= -3.26, Mmaj Ed= 4.67, case name= ULS_SC_Vx+_T-_Comp,

Case= 38, N Ed= 10.58, Vmaj Ed= 2.9, Mmaj Ed= -4.66, case name= ULS_SC_Vx+_T-_Tens,

Case= 39, N Ed= -1.24, Vmaj Ed= -3.27, Mmaj Ed= 4.65, case name= ULS_SC_T-_Comp,

Case= 40, N Ed= 10.26, Vmaj Ed= 2.89, Mmaj Ed= -4.64, case name= ULS_SC_T-_Tens,

Case= 41, N Ed= -1.18, Vmaj Ed= -2.96, Mmaj Ed= 4.19, case name= ULS_T-_SC_Comp,

Case= 42, N Ed= 17.73, Vmaj Ed= 2.62, Mmaj Ed= -4.27, case name= ULS_T-_SC_Tens,

Case= 43, N Ed= -11.23, Vmaj Ed= -1.03, Mmaj Ed= 1.99, case name= ULS_Vx-_T+_Comp,

Case= 44, N Ed= 0.64, Vmaj Ed= 1.36, Mmaj Ed= -1.99, case name= ULS_Vx-_T+_Tens,

Case= 45, N Ed= -1.56, Vmaj Ed= -1.08, Mmaj Ed= 2.07, case name= ULS_Vx-_T-_Comp,

Case= 46, N Ed= 12.23, Vmaj Ed= 1.37, Mmaj Ed= -2.08, case name= ULS_Vx-_T-_Tens,

Case= 47, N Ed= -12.26, Vmaj Ed= -3.29, Mmaj Ed= 4.71, case name= ULS_SC_Vx-_T+_Comp,

Case= 48, N Ed= 0.62, Vmaj Ed= 2.92, Mmaj Ed= -4.69, case name= ULS_SC_Vx-_T+_Tens,

Case= 49, N Ed= -2.06, Vmaj Ed= -3.28, Mmaj Ed= 4.63, case name= ULS_SC_Vx-_T-_Comp,

Case= 50, N Ed= 10.68, Vmaj Ed= 2.88, Mmaj Ed= -4.62, case name= ULS_SC_Vx-_T-_Tens,

Case= 51, N Ed= -13.02, Vmaj Ed= -2.97, Mmaj Ed= 4.33, case name= ULS_Vx+_SC_T+_Comp,

Case= 52, N Ed= 0.46, Vmaj Ed= 2.7, Mmaj Ed= -4.32, case name= ULS_Vx+_SC_T+_Tens,

Case= 53, N Ed= -1.11, Vmaj Ed= -2.96, Mmaj Ed= 4.25, case name= ULS_Vx+_SC_T-_Comp,

Case= 54, N Ed= 10.85, Vmaj Ed= 2.65, Mmaj Ed= -4.26, case name= ULS_Vx+_SC_T-_Tens,

Case= 55, N Ed= -12.39, Vmaj Ed= -3, Mmaj Ed= 4.26, case name= ULS_Vx-_SC_T+_Comp,

Case= 56, N Ed= 0.79, Vmaj Ed= 2.66, Mmaj Ed= -4.25, case name= ULS_Vx-_SC_T+_Tens,

Case= 57, N Ed= -2.54, Vmaj Ed= -2.98, Mmaj Ed= 4.18, case name= ULS_Vx-_SC_T-_Comp,

Case= 58, N Ed= 11.2, Vmaj Ed= 2.61, Mmaj Ed= -4.27, case name= ULS_Vx-_SC_T-_Tens,

Case= 59, N Ed= -11.83, Vmaj Ed= -3.06, Mmaj Ed= 4.73, case name= ULS_Vy+_SC_T+_Comp,

Case= 60, N Ed= 0.21, Vmaj Ed= -2.15, Mmaj Ed= 2.76, case name= ULS_Vy+_SC_T+_Tens,

Case= 61, N Ed= -10.48, Vmaj Ed= -3.05, Mmaj Ed= 4.65, case name= ULS_Vy+_SC_T-_Comp,

Case= 62, N Ed= 10.93, Vmaj Ed= -2.14, Mmaj Ed= 4.23, case name= ULS_Vy+_SC_T-_Tens,

Case= 63, N Ed= -10.6, Vmaj Ed= 1.29, Mmaj Ed= -1.69, case name= ULS_Vy+_T+_Comp,

Case= 64, N Ed= 0.18, Vmaj Ed= -0.18, Mmaj Ed= 0.24, case name= ULS_Vy+_T+_Tens,

Case= 65, N Ed= -10.35, Vmaj Ed= 1.3, Mmaj Ed= -1.71, case name= ULS_Vy+_T-_Comp,

Case= 66, N Ed= 12.09, Vmaj Ed= 0.98, Mmaj Ed= -1.23, case name= ULS_Vy+_T-_Tens,

Case= 67, N Ed= -11.93, Vmaj Ed= -3.33, Mmaj Ed= 4.99, case name= ULS_SC_Vy+_T+_Comp,

Case= 68, N Ed= 0.28, Vmaj Ed= -2.19, Mmaj Ed= 2.82, case name= ULS_SC_Vy+_T+_Tens,

Case= 69, N Ed= -6.71, Vmaj Ed= -3.32, Mmaj Ed= 4.91, case name= ULS_SC_Vy+_T-_Comp,

Case= 70, N Ed= 10.52, Vmaj Ed= -2.19, Mmaj Ed= 4.34, case name= ULS_SC_Vy+_T-_Tens,

Case= 71, N Ed= -12.89, Vmaj Ed= -2.99, Mmaj Ed= 4.37, case name= ULS_Vy-_SC_T+_Comp,

Case= 72, N Ed= 0.48, Vmaj Ed= 2.7, Mmaj Ed= -4.31, case name= ULS_Vy-_SC_T+_Tens,

Case= 73, N Ed= -10.91, Vmaj Ed= -2.98, Mmaj Ed= 4.35, case name= ULS_Vy-_SC_T-_Comp,

Case= 74, N Ed= 10.08, Vmaj Ed= 2.65, Mmaj Ed= -4.28, case name= ULS_Vy-_SC_T-_Tens,

Case= 75, N Ed= -11.61, Vmaj Ed= 1.36, Mmaj Ed= -1.95, case name= ULS_Vy-_T+_Comp,

Case= 76, N Ed= 0.14, Vmaj Ed= 0.74, Mmaj Ed= -1.64, case name= ULS_Vy-_T+_Tens,

Case= 77, N Ed= -10.25, Vmaj Ed= 1.38, Mmaj Ed= -2.03, case name= ULS_Vy-_T-_Comp,

Case= 78, N Ed= 11.21, Vmaj Ed= 1.15, Mmaj Ed= -1.39, case name= ULS_Vy-_T-_Tens,

Case= 79, N Ed= -12.74, Vmaj Ed= -3.28, Mmaj Ed= 4.75, case name= ULS_SC_Vy-_T+_Comp,

Case= 80, N Ed= 0.54, Vmaj Ed= 2.94, Mmaj Ed= -4.73, case name= ULS_SC_Vy-_T+_Tens,

Case= 81, N Ed= -6.94, Vmaj Ed= -3.27, Mmaj Ed= 4.7, case name= ULS_SC_Vy-_T-_Comp,

Case= 82, N Ed= 10.12, Vmaj Ed= 2.92, Mmaj Ed= -4.69, case name= ULS_SC_Vy-_T-_Tens,

Case= 83, N Ed= -0.14, Vmaj Ed= -1.53, Mmaj Ed= 2.5, case name= ULS_S1, Max_Comp,

Case= 84, N Ed= 15.41, Vmaj Ed= -1.84, Mmaj Ed= 3.12, case name= ULS_S1, Max_Tens,

Case= 85, N Ed= -15.11, Vmaj Ed= -1.93, Mmaj Ed= 2.99, case name= ULS_S1, Min_Comp,

Case= 86, N Ed= 0.06, Vmaj Ed= 1.59, Mmaj Ed= -2.78, case name= ULS_S1, Min_Tens,

Case= 87, N Ed= -0.05, Vmaj Ed= -1.43, Mmaj Ed= 2.34, case name= ULS_S2, Max_Comp,

Case= 88, N Ed= 20.95, Vmaj Ed= 1.85, Mmaj Ed= -3.17, case name= ULS_S2, Max_Tens,

Case= 89, N Ed= -20.65, Vmaj Ed= -1.94, Mmaj Ed= 2.97, case name= ULS_S2, Min_Comp,

Case= 90, N Ed= 0.02, Vmaj Ed= 1.5, Mmaj Ed= -2.78, case name= ULS_S2, Min_Tens,

γ M0= 1.1

γ M1= 1.1

γ M2= 1.25

# Members

Primary Member: IPE300

h = 300 mm

b = 150 mm

t f= 10.7 mm

t w= 7.1 mm

r = 15 mm

Material: S275

f y= 275 N/mm²

f u= 410 N/mm²

thk ≤: 16 mm

Only strength limit states are considered for member checks (e.g.: lateral/ torsional buckling not considered). Compr. buckling checked if options are present below member data.

Secondary Member: IPE160

h = 160 mm

b = 82 mm

t f= 7.4 mm

t w= 5 mm

r = 9 mm

Reinf. web pl. t  s= 0 mm

Material: S275

f y= 275 N/mm²

f u= 410 N/mm²

thk ≤: 16 mm

Option: Simplified calculation

Web class= 1

Flange class= 1

Beam class= 1 Table 5.2 EN1993-1-1

Web: Compr.

Flanges: Compr.

# Plate & Bolts

Bolt size: M16

d = 16 mm

d 0= 18 mm

Class: 8.8

f yb= 640 N/mm²

f ub= 800 N/mm²

α v= 0.6

Hole type: Normal

k red= 1

A = 201 mm²

A s= 157 mm²

Shear planes = 1

Conn. type: threads in shear plane

Shear cross section per bolt = 157 mm²

Plate material: S275

f yp= 275 N/mm²

f up= 410 N/mm²

thk ≤: 16 mm

# Geometry

bolt columns = 2

bolt rows = 3

total bolts = 6

plate thk = 10 mm

p horiz.= 80 mm

p vert.= 45 mm

a vert. beam top= 35 mm

a vert. beam bottom= 35 mm

depth pl.= 160 mm

width pl.= 154 mm

a vert. pl. top= 35 mm

a vert. pl. bottom= 35 mm

bolt gr. ecc. = 0 mm BoltGroupCenter distance from Conn. Axis

sec. ecc. = -10 mm Section near prim.

block shear k unif=1

# Weld, Notch & Stiffeners

Welding between plate and primary member:

f u,weld= 410 N/mm²

β w= 0.85

Web weld: double fillet

fillet number: 2

a web= 7.2 mm

l web= 127.2 mm

Welding the plate to flanges might increase capacity but it will also increase stiffness (not suggested by some literature)

Weld plate to beam flanges

Flange weld: single fillet

fillet number: 1

a flange= 7.2 mm

l flange= 118 mm

ecc. web weld= -10 mm Weld distance from Conn. Axis

Stiffeners on primary member:

No calculation is made on the stiffeners, they will only be added to the sketch.

t stiff.= 10 mm

Notch: No notch

Flange notch: No flange notch

# SumUp

Bolt shear: ok 1% (67)

Bolt min. distances: ok

Pl. bearing hor.: ok 0% (1)

Pl. bearing vert.: ok 1% (67)

Pl., bl. sh. vert.: ok 0% (67)

Pl., shear strong ax.: ok 1% (67)

Pl., shear weak ax.: ok 0% (1)

Sec. bending str.: ok 16% (67)

Sec. bending wk.: ok 0% (1)

Sec. axial: ok 4% (88)

Sec. shear str.: ok 2% (67)

Sec. shear wk.: ok 0% (1)

Sec. combined: ok 19% (67)

Prim. local web shear: ok 1% (67)

Web weld combined: ok 2% (88)

Flange weld combined: ok 23% (67)

Web weld σ┴: ok 1% (88)

Flange weld σ┴: ok 15% (67)

Bolt tension: ok 45% (82)

Bolt combined: ok 45% (67)

Moment cap.: ok 82% (67)

# Bolts

Case 67

Shear:

F v,Ed,hor,bolt gr.= 0 kN  , Vmin,Ed

F v,Ed,vert,bolt gr.= -3.33 kN  , Vmaj,Ed

M Ed,bolt gr.= 0 kN*m  , TEd

M ecc,Ed,bolt gr.= 0 kN*m  , Fv,Ed,vert,bolt gr.*bolt gr. ecc.+MEd,bolt gr.

β Lf= 1  =0.75≤ 1-90(mm)-15*16(mm)200*16(mm)≤1, 1-Lj-15d200d ; 0.75≤βLf≤1  (3.5)#00

F v,Rd,1 bolt= 60.29 kN  =0.6*800(N/mm²)*157(mm²)*11.25, αv fub A nsh.pl.γM2 (ta. 3.4)#00

F v,Rd,bolt gr.= 361.7 kN  , Bolt eccentr. calc. method: Elastic #26

F v,Ed,bolt gr.= 3.33 kN  =0(kN)2+-3.33(kN)2,

Bolt shear: ok 1%  , Fv,Ed,bolt gr.Fv,Rd,bolt gr.

Tension:

N Ed,bolt gr.= -11.93 kN  , NEd

M ┴Ed,bolt gr.= 4.99 kN*m  =4.99(kN*m)+-3.33(kN)/1*0(mm),

N Rd,1 bolt= 90.43 kN  =0.9*800(N/mm²)*157(mm²)1.25, 0.9 fub AsγM2  (ta 3.4) #00

N Rd,bolt gr.= 251.4 kN  , Moment calc. method: Eurocode/BS #42

N totEd,bolt gr.= 110.9 kN  , Moment calc. method: Eurocode/BS #42

Bolt tension: ok 44%  , Ntot,Ed,bolt gr.NRd,bolt gr.

Bolt combined: ok 45%  , Fv,Ed,1 boltFv,Rd,1 bolt+NEd,1 bolt1.4*NRd,1 bolt - Bolt 4

M ┴Rd,bolt gr.= 6.092 kN*m  , Moment calc. method: Eurocode/BS #42

Moment cap.: ok 82%  , M┴Ed,bolt gr.M┴Rd,bolt gr.

T-STUB DETAILS, bolt group 1

Positive Moment

Row 1: FT_Rd=135.4kN; Beam side; Individual; Mode 2; leff=184.4mm

Row 2: FT_Rd=60.02kN; Beam side; Group*: Rows 1 to 2: Mode 1; leff=229.4mm

Row 3: FT_Rd=56.02kN; Beam side; Group*: Rows 1 to 3: Mode 1; leff=295.2mm

Negative Moment

Row 3: FT_Rd=135.4kN; Beam side; Individual; Mode 2; leff=184.4mm

Row 2: FT_Rd=60.02kN; Beam side; Group*: Rows 3 to 2: Mode 1; leff=229.4mm

Row 1: FT_Rd=56.02kN; Beam side; Group*: Rows 3 to 1: Mode 1; leff=295.2mm

Details, Pos. Mom.

Row 1

Beam side:

Individual: Mode1: (4*(Min(2π*29.4, Max(4*29.4+1.25*37, α(26.9)(=6.3)*29.4)))*0.25*10^2*275/1.1)/29.4= 157.1kN;

Mode2: (2*(Min(2π*29.4, Max(4*29.4+1.25*37, α(26.9)(=6.3)*29.4)))*0.25*10^2*275/1.1+36.7*90.4*2*1E3)/(29.4+36.7)= 135.4kN;

Mode3: 90.4*2= 180.9kN;

Web tension: N.A.

Triang.limit not to be applied

Row 2

Beam side:

Individual: Mode1: (4*(Min(2π*29.4, 4*29.4+1.25*37))*0.25*10^2*275/1.1)/29.4= 139.4kN;

Mode2: (2*(Min(2π*29.4, 4*29.4+1.25*37))*0.25*10^2*275/1.1+36.7*90.4*2*1E3)/(29.4+36.7)= 131.5kN;

Mode3: 90.4*2= 180.9kN;

Web tension: 173.2kN

Group*: Rows 1 to 2: Mode1: (4*(Min(229.4, 229.4))*0.25*10^2*275/1.1)/29.4= 195.4kN;

Mode2: (2*(229.4)*0.25*10^2*275/1.1+36.7*90.4*4*1E3)/(29.4+36.7)= 244.4kN;

Mode3: 90.4*4= 361.7kN;

Web tension: N.A.

Triang.limit not to be applied

Row 3

Beam side:

Individual: Mode1: (4*(Min(2π*29.4, Max(4*29.4+1.25*37, α(26.9)(=6.3)*29.4)))*0.25*10^2*275/1.1)/29.4= 157.1kN;

Mode2: (2*(Min(2π*29.4, Max(4*29.4+1.25*37, α(26.9)(=6.3)*29.4)))*0.25*10^2*275/1.1+36.7*90.4*2*1E3)/(29.4+36.7)= 135.4kN;

Mode3: 90.4*2= 180.9kN;

Web tension: N.A.

Group*: Rows 2 to 3: Mode1: (4*(Min(229.4, 229.4))*0.25*10^2*275/1.1)/29.4= 195.4kN;

Mode2: (2*(229.4)*0.25*10^2*275/1.1+36.7*90.4*4*1E3)/(29.4+36.7)= 244.4kN;

Mode3: 90.4*4= 361.7kN;

Web tension: N.A.

Group*: Rows 1 to 3: Mode1: (4*(Min(295.2, 295.2))*0.25*10^2*275/1.1)/29.4= 251.4kN;

Mode2: (2*(295.2)*0.25*10^2*275/1.1+36.7*90.4*6*1E3)/(29.4+36.7)= 357.3kN;

Mode3: 90.4*6= 542.6kN;

Web tension: N.A.

Triang.limit not to be applied

Details, Neg. Mom.

Row 3

Beam side:

Individual: Mode1: (4*(Min(2π*29.4, Max(4*29.4+1.25*37, α(26.9)(=6.3)*29.4)))*0.25*10^2*275/1.1)/29.4= 157.1kN;

Mode2: (2*(Min(2π*29.4, Max(4*29.4+1.25*37, α(26.9)(=6.3)*29.4)))*0.25*10^2*275/1.1+36.7*90.4*2*1E3)/(29.4+36.7)= 135.4kN;

Mode3: 90.4*2= 180.9kN;

Web tension: N.A.

Triang.limit not to be applied

Row 2

Beam side:

Individual: Mode1: (4*(Min(2π*29.4, 4*29.4+1.25*37))*0.25*10^2*275/1.1)/29.4= 139.4kN;

Mode2: (2*(Min(2π*29.4, 4*29.4+1.25*37))*0.25*10^2*275/1.1+36.7*90.4*2*1E3)/(29.4+36.7)= 131.5kN;

Mode3: 90.4*2= 180.9kN;

Web tension: 173.2kN

Group*: Rows 3 to 2: Mode1: (4*(Min(229.4, 229.4))*0.25*10^2*275/1.1)/29.4= 195.4kN;

Mode2: (2*(229.4)*0.25*10^2*275/1.1+36.7*90.4*4*1E3)/(29.4+36.7)= 244.4kN;

Mode3: 90.4*4= 361.7kN;

Web tension: N.A.

Triang.limit not to be applied

Row 1

Beam side:

Individual: Mode1: (4*(Min(2π*29.4, Max(4*29.4+1.25*37, α(26.9)(=6.3)*29.4)))*0.25*10^2*275/1.1)/29.4= 157.1kN;

Mode2: (2*(Min(2π*29.4, Max(4*29.4+1.25*37, α(26.9)(=6.3)*29.4)))*0.25*10^2*275/1.1+36.7*90.4*2*1E3)/(29.4+36.7)= 135.4kN;

Mode3: 90.4*2= 180.9kN;

Web tension: N.A.

Group*: Rows 2 to 1: Mode1: (4*(Min(229.4, 229.4))*0.25*10^2*275/1.1)/29.4= 195.4kN;

Mode2: (2*(229.4)*0.25*10^2*275/1.1+36.7*90.4*4*1E3)/(29.4+36.7)= 244.4kN;

Mode3: 90.4*4= 361.7kN;

Web tension: N.A.

Group*: Rows 3 to 1: Mode1: (4*(Min(295.2, 295.2))*0.25*10^2*275/1.1)/29.4= 251.4kN;

Mode2: (2*(295.2)*0.25*10^2*275/1.1+36.7*90.4*6*1E3)/(29.4+36.7)= 357.3kN;

Mode3: 90.4*6= 542.6kN;

Web tension: N.A.

Triang.limit not to be applied

*: Group values are from total values, that is to find final value upper (or lower) row values should be detracted

# Plate bearing

Case: 67

Tot. ref. thk = 10

p 1,min.= 39.6 mm

e 1,min.= 21.6 mm

p 2,min.= 43.2 mm

e 2,min.= 21.6 mm

p 1,opt.= 63 mm

e 1,opt.= 54 mm

p 2,opt.= 54 mm

e 2,opt.= 27 mm

Horizontal forces

p 1,hpl= 80 mm ok

e 1,hpl= 37 mm ok

p 2,hpl= 45 mm ok

e 2,hpl= 54 mm ok

α b,hpl= 0.685

α d,hpl= 0.685

k 1,hpl= 1.8

Vertical forces

p 1,vpl= 45 mm ok

e 1,vpl= 54 mm ok

p 2,vpl= 80 mm ok

e 2,vpl= 37 mm ok

α b,vpl= 0.583

α d,vpl= 0.583

k 1,pl= 2.5

F b,Rd,h,pl= 64.73 kN  =1.233*1*410(N/mm²)*16(mm)*10(mm)/1.25, k1,hpl αb,hpl kred,hor. fu d tγM2 (ta. 3.4)#00

F b,Ed,h,pl= 0 kN Bolt 6

F b,Rd,v,pl= 76.53 kN  =1.458*1*410(N/mm²)*16(mm)*10(mm)/1.25, k1,vpl αb,vpl kred,vert. fu d tγM2 (ta. 3.4)#00

F b,Ed,v,pl= 0.555 kN Bolt 6

Plate bear. h.: ok 0%

Plate bear. v.: ok 1%

# Beam bearing

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

Not applicable check.

# Plate block shear (block tearing)

Case: 67

k unif= 1

Plate block sh. - V:

Plate block shear (block tearing) - V = Vvert,sd,bolt gr.

Mode 4:

A nt= ∞ mm²

A nv= 1600 mm²

V eff,Rd4= ∞ kN

Mode 5:

A nt= ∞ mm²

A nv= 1600 mm²

V eff,Rd5= ∞ kN

V eff,Rd V Pl= ∞ kN  , min (Veff,Rd4; Veff,Rd5)

V eff,Ed V Pl= 3.33 kN  , Vmaj,Ed

ok 0%

# Plate stress

Case: 67

Only shear is checked because the other actions are checked through T-stub verification.

V Rd,str,pl= 230.9 kN  =1600(mm²)*275(N/mm²)3*1.1, Av fy3 γM0 (6.18) #01

V Ed,str,pl= 1.665 kN  , Vmaj,Ed/2

Shear str: ok 1%  , VEd,str,plVRd,str,pl

V Rd,wk,pl= 222.3 kN  =1540(mm²)*275(N/mm²)3*1.1, Avfy3 γM0 (6.18) #01

V Ed,wk,pl= 0 kN  , Vmin,Ed/2

Shear wk: ok 0%  , VEd,wk,plVRd,wk,pl

# Beam stress

Case: 67

N Rd,sec= 502.3 kN  =min1*2009(mm²)275(N/mm²)1.1;2009(mm²)275(N/mm²)1.1, minχAfyγM1;AfyγM0 (6.47, 6.10)#01

N Ed,sec= -11.93 kN  NEd

Axial: ok 2%  , NEd,secNRd,sec

M Rd,str,sec= 30.96 kN*m  =1.239*105(mm³)*275(N/mm²)*1-01.1, Wpl (1-ρ) fy 1γM0 (6.13, 6.29)#01

M Ed,str,sec= 5.023 kN*m  , Vmaj,Ed*ecc.+MEd,str

Bending str: ok 16%  , MEd,str,secMRd,str,sec

V Rd,str,sec= 139.4 kN  =965.7(mm²)*275(N/mm²)3*1.1, Av fy3 γM0 (6.18) #01

V Ed,str,sec= 3.33 kN  , Vmaj,Ed

Shear str: ok 2%  , VEd,str,secVRd,str,sec

M Rd,wk,sec= 6.447 kN*m  =2.579*104(mm³)*275(N/mm²)1.1, Wpl fyγM0 (6.13) #01

M Ed,wk,sec= 0 kN*m  Vmin,Ed*ecc.

Bending wk: ok 0%  , MEd,wk,secMRd,wk,sec

V Rd,wk,sec= 185.2 kN  =1283(mm²)*275(N/mm²)3*1.1, Avfy3 γM0 (6.18) #01

V Ed,wk,sec= 0 kN  , VEd,wk,sec

Shear wk: ok 0%  , VEd,wk,secVRd,wk,sec

Combined: ok 19%  =0.024+0.162+0, NEdNRd + MEd,strMRd,str + MEd,wkMRd,wk (6.2)#01

# Other checks

Case: 67

Prim. local shear:

A v= 1650 mm²  =(35(mm)+(3-1)*45(mm)+40(mm))*10(mm), check 10 #41

V Rd,loc= 238.2 kN  =1650(mm²)*275(N/mm²)3*1.1, Avfy3 γM0 6.5 check 10* #41

V Ed,loc= 1.665 kN  , Vmaj,Ed/2

Prim. local shear: ok 1%  =1.665(kN)238.2(kN), FEd,localshearFRd,localshear

# Welding

Case: 67

Web weld

A throat= 1832 mm²  =254.4(mm)*7.2(mm), aweb lweb

N Ed= 6.189 kN

V Ed,str= 3.33 kN

V Ed,wk= 0 kN

M Ed,str= 0 kN*m  , Vmaj,Ed*ecc.

M Ed,wk= 0 kN*m  , Mmin,Ed

T Ed= 0 kN*m  , TEd

W R,str= 3.883E+04 mm³  , lweb2 aw fillet number 1/6

W R,wk= 2198 mm³  , fillet number lweb aweb26

W R,T= 2.235E+04 mm³  , Aw (tw,sec+a)

σ ┴= 2.389 N/mm²  (=6.189(kN)1832(mm²)+0(kN*m)3.883*104(mm³)+0(kN*m)2198(mm³))*cos45°+(=0(kN)1832(mm²))*sin45°, (NEdAthroat+MEd,strWR,str+MEd,wkWR,wk)*cos45°+(VEd,wkAthroat)*sin45°

τ ┴= -2.389 N/mm²  -(=6.189(kN)1832(mm²)+0(kN*m)3.883*104(mm³)+0(kN*m)2198(mm³))*sin45°+(=0(kN)1832(mm²))*cos45°, -(NEdAthroat+MEd,strWR,str+MEd,wkWR,wk)*sin45°+(VEd,wkAthroat)*cos45°

τ ||= 1.818 N/mm²  =3.33(kN)1832(mm²)+0(kN*m)2.235*104(mm³), VEd,strAthroat+TEdWR,T

σ v= 5.723 N/mm²  =(2.389(N/mm²))2+3(-2.389(N/mm²))2+(1.818(N/mm²))2, σ┴2 + 3τ┴2 + τ║2 (4.1)#00

σ Rd= 385.9 N/mm²  =410(N/mm²)0.85*1.25, fuβw γM2 (4.1)#00

Weld comb.: ok 1%  5.723(N/mm²)385.9(N/mm²), σw,vσw,Rd

Weld σ┴: ok 1%  2.389(N/mm²)295.2(N/mm²), σ┴0.9 fu/γM2 (4.1)#00

Flange weld

A throat= 849.6 mm²  =118(mm)*7.2(mm), aflange lflange

N Ed= 5.741 kN

V Ed,str= 0 kN

V Ed,wk= 0 kN

M Ed,str= 4.957 kN*m  , Mmaj,Ed+Vmaj,Ed*ecc.

M Ed,wk= 0 kN*m  , Vmin,Ed*ecc.

T Ed= 0 kN*m  , TEd

W R,str= 9.021E+04 mm³  , Elastic Equiv. Section

W R,wk= 0 mm³  , Elastic Equiv. Section

W R,T= 0 mm³  , width*depth*fillet number*throat thkflange

σ ┴= 43.63 N/mm²  (=5.741(kN)849.6(mm²)+4.957(kN*m)9.021*104(mm³)+0(kN*m)0(mm³))*cos45°+(=0(kN)849.6(mm²))*sin45°, (NEdAthroat+MEd,strWR,str+MEd,wkWR,wk)*cos45°+(VEd,wkAthroat)*sin45°

τ ┴= -43.63 N/mm²  -(=5.741(kN)849.6(mm²)+4.957(kN*m)9.021*104(mm³)+0(kN*m)0(mm³))*sin45°+(=0(kN)849.6(mm²))*cos45°, -(NEdAthroat+MEd,strWR,str+MEd,wkWR,wk)*sin45°+(VEd,wkAthroat)*cos45°

τ ||= 0 N/mm²  =0(kN)849.6(mm²), VEd,strAthroat

σ v= 87.26 N/mm²  =(43.63(N/mm²))2+3(-43.63(N/mm²))2+(0(N/mm²))2, σ┴2 + 3τ┴2 + τ║2 (4.1)#00

σ Rd= 385.9 N/mm²  =410(N/mm²)0.85*1.25, fuβw γM2 (4.1)#00

Weld comb.: ok 23%  87.26(N/mm²)385.9(N/mm²), σw,vσw,Rd

Weld σ┴: ok 15%  43.63(N/mm²)295.2(N/mm²), σ┴0.9 fu/γM2 (4.1)#00

# Comments/Notes

Manual update turned on, click Update Calcs to see results

Env. output from joints 43, 43, 45, 45, 46, 46, 47, 47, 48, 48, 49, 49, 51, 51, 52, 52, 53, 53, 55, 55, 59, 59, 65, 65, 69, 69, 71, 71, 73, 73, 74, 74, 75, 75, 77, 77, 79, 79, 81, 81, 83, 83, 87, 87, 93, 93, 95, 95, 96, 96, 97, 97, 98, 98, 99, 99, 101, 101, 103, 103, 105, 105, 107, 107, 110, 110, 113, 113, 115, 115, 116, 116, 117, 117, 118, 118, 119, 119, 121, 121, 123, 123, 124, 124, 125, 125, 127, 127, 130, 130, 132, 132, 133, 133, 134, 134, 135, 135, 136, 136, 137, 137, 138, 138, 139, 139, 140, 140, 143, 143, 146, 146, 148, 148, 149, 149, 150, 150, 151, 151, 152, 152, 153, 153, 156, 156, 158, 158, 160, 160, 162, 162, 165, 165, 167, 167, 168, 168, 169, 169, 170, 170, 171, 171, 172, 172, 173, 173, 174, 174, 175, 175, 177, 177, 181, 181, 182, 182, 183, 183, 184, 184, 185, 185, 186, 186, 187, 187, 188, 188, 189, 189, 190, 190, 191, 191 sec. members 11, 12, 18, 37, 37, 43, 43, 46, 46, 47, 47, 48, 48, 49, 49, 50, 50, 51, 51, 52, 53, 54, 55, 56, 57, 58, 58, 59, 59, 60, 60, 61, 61, 62, 62, 63, 63, 66, 66, 67, 67, 68, 69, 70, 71, 72, 75, 77, 77, 80, 80, 81, 81, 82, 82, 83, 83, 84, 84, 85, 85, 87, 87, 88, 89, 90, 91, 92, 93, 94, 94, 95, 95, 96, 96, 97, 97, 98, 98, 99, 99, 100, 100, 101, 101, 102, 103, 104, 105, 106, 107, 108, 108, 122, 122, 126, 126, 145, 145, 146, 146, 152, 152, 155, 155, 156, 156, 161, 162, 163, 168, 193, 194, 196, 196, 197, 197, 198, 198, 199, 199, 200, 200, 201, 201, 209, 209, 210, 210, 219, 220, 227, 237, 240, 243, 244, 244, 249, 249, 251, 251, 252, 252, 253, 253, 254, 254, 255, 255, 257, 257, 258, 259, 260, 263, 265, 265, 266, 266, 268, 268, 269, 269, 271, 271, 272, 272, 273, 273, 274, 274, 275, 275, 277, 277, 278 file MODEL_V1.0.sdb.

The exported load combinations seem to have relevant minor axis moment actions that SCS cannot manage. Please check carefully your model releases: this connection is supposed to be a hinge and SCS cannot take weak axis moments

Env. output from joints 43, 43, 45, 45, 46, 46, 47, 47, 48, 48, 49, 49, 51, 51, 52, 52, 53, 53, 55, 55, 59, 59, 65, 65, 69, 69, 71, 71, 73, 73, 74, 74, 75, 75, 77, 77, 79, 79, 81, 81, 83, 83, 87, 87, 93, 93, 95, 95, 96, 96, 97, 97, 98, 98, 99, 99, 101, 101, 103, 103, 105, 105, 107, 107, 110, 110, 113, 113, 115, 115, 116, 116, 117, 117, 118, 118, 119, 119, 121, 121, 123, 123, 124, 124, 125, 125, 127, 127, 130, 130, 132, 132, 133, 133, 134, 134, 135, 135, 136, 136, 137, 137, 138, 138, 139, 139, 140, 140, 143, 143, 146, 146, 148, 148, 149, 149, 150, 150, 151, 151, 152, 152, 153, 153, 156, 156, 158, 158, 160, 160, 162, 162, 165, 165, 167, 167, 168, 168, 169, 169, 170, 170, 171, 171, 172, 172, 173, 173, 174, 174, 175, 175, 177, 177, 181, 181, 182, 182, 183, 183, 184, 184, 185, 185, 186, 186, 187, 187, 188, 188, 189, 189, 190, 190, 191, 191 sec. members 11, 12, 18, 37, 37, 43, 43, 46, 46, 47, 47, 48, 48, 49, 49, 50, 50, 51, 51, 52, 53, 54, 55, 56, 57, 58, 58, 59, 59, 60, 60, 61, 61, 62, 62, 63, 63, 66, 66, 67, 67, 68, 69, 70, 71, 72, 75, 77, 77, 80, 80, 81, 81, 82, 82, 83, 83, 84, 84, 85, 85, 87, 87, 88, 89, 90, 91, 92, 93, 94, 94, 95, 95, 96, 96, 97, 97, 98, 98, 99, 99, 100, 100, 101, 101, 102, 103, 104, 105, 106, 107, 108, 108, 122, 122, 126, 126, 145, 145, 146, 146, 152, 152, 155, 155, 156, 156, 161, 162, 163, 168, 193, 194, 196, 196, 197, 197, 198, 198, 199, 199, 200, 200, 201, 201, 209, 209, 210, 210, 219, 220, 227, 237, 240, 243, 244, 244, 249, 249, 251, 251, 252, 252, 253, 253, 254, 254, 255, 255, 257, 257, 258, 259, 260, 263, 265, 265, 266, 266, 268, 268, 269, 269, 271, 271, 272, 272, 273, 273, 274, 274, 275, 275, 277, 277, 278 file MODEL_V2.0.sdb.

References

#00: EN 1993-1-8: 2005, Design of steel structures, Design of joints, Cen

#01: EN 1993-1-1: 2001, Design of steel structures, General structural rules, Cen

#26: AISC Steel Construction Manual

#41: Joints in steel constructions, Simple Connections, SCI and BCSA, 2002

#42: Joints in steel constructions, Moment Connections, SCI and BCSA, 1995