# C37_A.em3

Bolted end plate

# General

This connection can bear any kind of forces and it's the state of the art when column-beam connection on main column axis must transmit consistent bending moment.

Standard: Eurocode

Bolt eccentr. calc. method: Elastic

Connection axis: Column-Plate contact point

Category for shear: Bearing type

Moment calc. method: Eurocode/BS #42

Top configuration: Extended 1 row + stiffener

Bottom configuration: Flush haunch

Seismicity: Joint is NOT dissipative

# Loads

Units: kN, kN*m

Case= 1, N Ed= -29.79, Vmaj Ed= 215.14, Vmin Ed= -0.16, Mmaj Ed= 255.58, Mmin Ed= 0.3, T Ed= -1.18, case name= ULS_SC_Comp,

Case= 2, N Ed= 4.71, Vmaj Ed= 224.01, Vmin Ed= -0.18, Mmaj Ed= 304.65, Mmin Ed= -0.1, T Ed= 0.87, case name= ULS_SC_Tens,

Case= 3, N Ed= -33.72, Vmaj Ed= 216.5, Vmin Ed= -6.47, Mmaj Ed= 260.47, Mmin Ed= 2.54, T Ed= -1.22, case name= ULS_SC_Vx+_Comp,

Case= 4, N Ed= 4.73, Vmaj Ed= 226.57, Vmin Ed= 4.9, Mmaj Ed= 313.69, Mmin Ed= 1.66, T Ed= 1.11, case name= ULS_SC_Vx+_Tens,

Case= 5, N Ed= -33.85, Vmaj Ed= 191.91, Vmin Ed= -10.78, Mmaj Ed= 232.06, Mmin Ed= 4.21, T Ed= -1.1, case name= ULS_Vx+_SC_Comp,

Case= 6, N Ed= 6.71, Vmaj Ed= 200.96, Vmin Ed= 8.18, Mmaj Ed= 285.49, Mmin Ed= 2.8, T Ed= 0.97, case name= ULS_Vx+_SC_Tens,

Case= 7, N Ed= -4.61, Vmaj Ed= 58.09, Vmin Ed= 0.18, Mmaj Ed= 82.49, Mmin Ed= -0.42, T Ed= 0.31, case name= ULS_Vx+_Comp,

Case= 8, N Ed= 8.55, Vmaj Ed= 58.66, Vmin Ed= -10.76, Mmaj Ed= 80.48, Mmin Ed= 4.17, T Ed= -0.35, case name= ULS_Vx+_Tens,

Case= 9, N Ed= -24.52, Vmaj Ed= 213.98, Vmin Ed= -0.15, Mmaj Ed= 251.36, Mmin Ed= 0.66, T Ed= -1.14, case name= ULS_SC_Vx-_Comp,

Case= 10, N Ed= 5.78, Vmaj Ed= 221.81, Vmin Ed= 0.08, Mmaj Ed= 296.91, Mmin Ed= 0.06, T Ed= -0.89, case name= ULS_SC_Vx-_Tens,

Case= 11, N Ed= -18.52, Vmaj Ed= -186.58, Vmin Ed= 0.14, Mmaj Ed= 216.59, Mmin Ed= 0.93, T Ed= -0.96, case name= ULS_Vx-_SC_Comp,

Case= 12, N Ed= 6.07, Vmaj Ed= 193.02, Vmin Ed= 0.04, Mmaj Ed= 257.53, Mmin Ed= 0.05, T Ed= -0.88, case name= ULS_Vx-_SC_Tens,

Case= 13, N Ed= -5.81, Vmaj Ed= -54.96, Vmin Ed= 0.23, Mmaj Ed= 68, Mmin Ed= 0.14, T Ed= -0.32, case name= ULS_Vx-_Comp,

Case= 14, N Ed= 16.08, Vmaj Ed= -58.81, Vmin Ed= 0.11, Mmaj Ed= 81.82, Mmin Ed= 0.7, T Ed= 0.31, case name= ULS_Vx-_Tens,

Case= 15, N Ed= -49.48, Vmaj Ed= 195.53, Vmin Ed= -0.43, Mmaj Ed= 247.51, Mmin Ed= 0.92, T Ed= -1.04, case name= ULS_Vy+_SC_Comp,

Case= 16, N Ed= 17.67, Vmaj Ed= 208.1, Vmin Ed= -0.27, Mmaj Ed= 311.71, Mmin Ed= 0.16, T Ed= 0.96, case name= ULS_Vy+_SC_Tens,

Case= 17, N Ed= -20.25, Vmaj Ed= 60.11, Vmin Ed= -0.27, Mmaj Ed= 91.94, Mmin Ed= 0.69, T Ed= -0.29, case name= ULS_Vy+_Comp,

Case= 18, N Ed= 12.52, Vmaj Ed= 65.23, Vmin Ed= -0.12, Mmaj Ed= 108.71, Mmin Ed= -0.16, T Ed= 0.27, case name= ULS_Vy+_Tens,

Case= 19, N Ed= -43.09, Vmaj Ed= 219.06, Vmin Ed= -0.33, Mmaj Ed= 269.91, Mmin Ed= 0.65, T Ed= -1.19, case name= ULS_SC_Vy+_Comp,

Case= 20, N Ed= 12.74, Vmaj Ed= 230.86, Vmin Ed= -0.24, Mmaj Ed= 329.42, Mmin Ed= 0.12, T Ed= 1.11, case name= ULS_SC_Vy+_Tens,

Case= 21, N Ed= -16.99, Vmaj Ed= -189.7, Vmin Ed= 0.11, Mmaj Ed= 232.33, Mmin Ed= 0.36, T Ed= -1.01, case name= ULS_Vy-_SC_Comp,

Case= 22, N Ed= 12.27, Vmaj Ed= -129.36, Vmin Ed= 0.21, Mmaj Ed= 143.95, Mmin Ed= -0.63, T Ed= -0.77, case name= ULS_Vy-_SC_Tens,

Case= 23, N Ed= -18.42, Vmaj Ed= -62.75, Vmin Ed= 0.14, Mmaj Ed= 99.92, Mmin Ed= -0.18, T Ed= 0.27, case name= ULS_Vy-_Comp,

Case= 24, N Ed= 26.09, Vmaj Ed= -61.93, Vmin Ed= 0.37, Mmaj Ed= 92.28, Mmin Ed= -0.87, T Ed= 0.28, case name= ULS_Vy-_Tens,

Case= 25, N Ed= -18.51, Vmaj Ed= 217.83, Vmin Ed= -0.07, Mmaj Ed= 281.79, Mmin Ed= 0.35, T Ed= -1.17, case name= ULS_SC_Vy-_Comp,

Case= 26, N Ed= 3.56, Vmaj Ed= -147.39, Vmin Ed= -0.14, Mmaj Ed= 162.01, Mmin Ed= -0.14, T Ed= 0.85, case name= ULS_SC_Vy-_Tens,

Case= 27, N Ed= -0.36, Vmaj Ed= -17.53, Vmin Ed= 3.97, Mmaj Ed= -7.26, Mmin Ed= 1.33, T Ed= 0.88, case name= ENV_ULS, Max_Comp,

Case= 28, N Ed= 220.63, Vmaj Ed= 260.17, Vmin Ed= 27.07, Mmaj Ed= 54.85, Mmin Ed= 4.2, T Ed= 1.2, case name= ENV_ULS, Max_Tens,

Case= 29, N Ed= -264.29, Vmaj Ed= -214.96, Vmin Ed= -27.09, Mmaj Ed= 320.2, Mmin Ed= -2.99, T Ed= -1.21, case name= ENV_ULS, Min,

Case= 30, N Ed= -69.39, Vmaj Ed= 224.46, Vmin Ed= 0.12, Mmaj Ed= 306.87, Mmin Ed= 0.19, T Ed= -1.21, case name= ULS_SC_T+,

Case= 31, N Ed= -119.21, Vmaj Ed= 197.44, Vmin Ed= 0.19, Mmaj Ed= 274.12, Mmin Ed= -0.39, T Ed= -1.07, case name= ULS_T+_SC,

Case= 32, N Ed= -77.45, Vmaj Ed= 58.87, Vmin Ed= -10.73, Mmaj Ed= 84.71, Mmin Ed= 4.11, T Ed= -0.38, case name= ULS_Vx+_T+,

Case= 33, N Ed= -1.33, Vmaj Ed= -2.53, Vmin Ed= -0.01, Mmaj Ed= -4.84, Mmin Ed= -0.06, case name= ULS_Vx+_T-_Comp,

Case= 34, N Ed= 70.76, Vmaj Ed= 58.46, Vmin Ed= -10.78, Mmaj Ed= 80.28, Mmin Ed= 4.22, T Ed= -0.33, case name= ULS_Vx+_T-_Tens,

Case= 35, N Ed= -70.89, Vmaj Ed= 227.03, Vmin Ed= -6.45, Mmaj Ed= 315.9, Mmin Ed= 2.49, T Ed= -1.25, case name= ULS_SC_Vx+_T+,

Case= 36, N Ed= -8.19, Vmaj Ed= 148.33, Vmin Ed= -0.21, Mmaj Ed= 217.67, Mmin Ed= 0.57, T Ed= 1.18, case name= ULS_SC_Vx+_T-_Comp,

Case= 37, N Ed= 77.32, Vmaj Ed= 226.12, Vmin Ed= -6.5, Mmaj Ed= 311.47, Mmin Ed= 2.6, T Ed= -1.2, case name= ULS_SC_Vx+_T-_Tens,

Case= 38, N Ed= -4.27, Vmaj Ed= 146.33, Vmin Ed= -0.18, Mmaj Ed= 210.69, Mmin Ed= 0.57, T Ed= 1.14, case name= ULS_SC_T-_Comp,

Case= 39, N Ed= 78.82, Vmaj Ed= 223.55, Vmin Ed= -0.36, Mmaj Ed= 302.44, Mmin Ed= -0.23, T Ed= -1.15, case name= ULS_SC_T-_Tens,

Case= 40, N Ed= -0.59, Vmaj Ed= -8.78, Vmin Ed= 0.12, Mmaj Ed= 29.9, Mmin Ed= 0.78, case name= ULS_T-_SC_Comp,

Case= 41, N Ed= 127.81, Vmaj Ed= 195.93, Vmin Ed= -0.48, Mmaj Ed= 266.75, Mmin Ed= 0.5, T Ed= -0.99, case name= ULS_T-_SC_Tens,

Case= 42, N Ed= -73.19, Vmaj Ed= -58.61, Vmin Ed= 0.38, Mmaj Ed= 81.6, Mmin Ed= -0.48, T Ed= -0.33, case name= ULS_Vx-_T+_Comp,

Case= 43, N Ed= 1.74, Vmaj Ed= -6.29, Vmin Ed= 0.05, Mmaj Ed= 20.7, Mmin Ed= 0.35, case name= ULS_Vx-_T+_Tens,

Case= 44, N Ed= 75.03, Vmaj Ed= -59.02, Vmin Ed= -0.18, Mmaj Ed= 82.04, Mmin Ed= 1.05, T Ed= -0.31, case name= ULS_Vx-_T-,

Case= 45, N Ed= -69.38, Vmaj Ed= 222.26, Vmin Ed= 0.13, Mmaj Ed= 299.12, Mmin Ed= 0.31, T Ed= -1.17, case name= ULS_SC_Vx-_T+,

Case= 46, N Ed= -0.62, Vmaj Ed= -10.29, Vmin Ed= 0.15, Mmaj Ed= 40.19, Mmin Ed= 1.01, case name= ULS_SC_Vx-_T-_Comp,

Case= 47, N Ed= 79.88, Vmaj Ed= 221.35, Vmin Ed= -0.36, Mmaj Ed= 294.7, Mmin Ed= 0.27, T Ed= -1.12, case name= ULS_SC_Vx-_T-_Tens,

Case= 48, N Ed= -72.3, Vmaj Ed= 201.42, Vmin Ed= -10.75, Mmaj Ed= 287.7, Mmin Ed= 4.16, T Ed= -1.12, case name= ULS_Vx+_SC_T+,

Case= 49, N Ed= -8.32, Vmaj Ed= 133.44, Vmin Ed= -0.23, Mmaj Ed= 200.79, Mmin Ed= 0.64, T Ed= 1.05, case name= ULS_Vx+_SC_T-_Comp,

Case= 50, N Ed= 75.91, Vmaj Ed= 200.51, Vmin Ed= -10.8, Mmaj Ed= 283.28, Mmin Ed= 4.26, T Ed= -1.07, case name= ULS_Vx+_SC_T-_Tens,

Case= 51, N Ed= -69.95, Vmaj Ed= 193.47, Vmin Ed= -0.18, Mmaj Ed= 259.74, Mmin Ed= 0.58, T Ed= -0.99, case name= ULS_Vx-_SC_T+,

Case= 52, N Ed= 80.18, Vmaj Ed= 192.56, Vmin Ed= -0.34, Mmaj Ed= 255.32, Mmin Ed= 1.29, T Ed= 0.96, case name= ULS_Vx-_SC_T-,

Case= 53, N Ed= -88.23, Vmaj Ed= 208.55, Vmin Ed= -0.23, Mmaj Ed= 313.92, Mmin Ed= 0.57, T Ed= -1.07, case name= ULS_Vy+_SC_T+,

Case= 54, N Ed= -23.95, Vmaj Ed= 143.31, Vmin Ed= -0.22, Mmaj Ed= 236.78, Mmin Ed= 1.27, T Ed= 1, case name= ULS_Vy+_SC_T-_Comp,

Case= 55, N Ed= 91.78, Vmaj Ed= 207.64, Vmin Ed= -0.63, Mmaj Ed= 309.5, Mmin Ed= 0.36, T Ed= -1.02, case name= ULS_Vy+_SC_T-_Tens,

Case= 56, N Ed= -79.28, Vmaj Ed= 65.68, Vmin Ed= 0.21, Mmaj Ed= 110.93, Mmin Ed= 0.34, T Ed= -0.32, case name= ULS_Vy+_T+,

Case= 57, N Ed= -8.92, Vmaj Ed= -1.08, Vmin Ed= 0.15, Mmaj Ed= -14.71, Mmin Ed= 1.04, case name= ULS_Vy+_T-_Comp,

Case= 58, N Ed= 86.63, Vmaj Ed= 64.77, Vmin Ed= -0.47, Mmaj Ed= 106.5, Mmin Ed= 0.25, T Ed= -0.27, case name= ULS_Vy+_T-_Tens,

Case= 59, N Ed= -80.35, Vmaj Ed= 231.31, Vmin Ed= 0.17, Mmaj Ed= 331.63, Mmin Ed= 0.37, T Ed= -1.21, case name= ULS_SC_Vy+_T+,

Case= 60, N Ed= -17.57, Vmaj Ed= 154.26, Vmin Ed= -0.21, Mmaj Ed= 239.26, Mmin Ed= 1, T Ed= 1.15, case name= ULS_SC_Vy+_T-_Comp,

Case= 61, N Ed= 86.85, Vmaj Ed= 230.4, Vmin Ed= -0.53, Mmaj Ed= 327.21, Mmin Ed= 0.27, T Ed= -1.16, case name= ULS_SC_Vy+_T-_Tens,

Case= 62, N Ed= -87.38, Vmaj Ed= -189.5, Vmin Ed= 0.41, Mmaj Ed= 234.54, Mmin Ed= -0.99, T Ed= -1.04, case name= ULS_Vy-_SC_T+,

Case= 63, N Ed= 71.3, Vmaj Ed= -189.9, Vmin Ed= -0.27, Mmaj Ed= 230.12, Mmin Ed= 0.5, T Ed= -0.99, case name= ULS_Vy-_SC_T-,

Case= 64, N Ed= -92.53, Vmaj Ed= -62.29, Vmin Ed= 0.57, Mmaj Ed= 98.77, Mmin Ed= -0.29, T Ed= -0.29, case name= ULS_Vy-_T+_Comp,

Case= 65, N Ed= 4.39, Vmaj Ed= 21.99, Vmin Ed= -0.18, Mmaj Ed= 30.17, Mmin Ed= -1.22, T Ed= 0.3, case name= ULS_Vy-_T+_Tens,

Case= 66, N Ed= 80.26, Vmaj Ed= -63.21, Vmin Ed= -0.24, Mmaj Ed= 101.06, Mmin Ed= -0.51, T Ed= 0.27, case name= ULS_Vy-_T-,

Case= 67, N Ed= -79.94, Vmaj Ed= 218.28, Vmin Ed= 0.26, Mmaj Ed= 284, Mmin Ed= -0.63, T Ed= -1.2, case name= ULS_SC_Vy-_T+,

Case= 68, N Ed= 68.28, Vmaj Ed= 217.38, Vmin Ed= -0.3, Mmaj Ed= 279.58, Mmin Ed= 0.49, T Ed= -1.15, case name= ULS_SC_Vy-_T-,

Case= 69, N Ed= -3.84, Vmaj Ed= -101.07, Vmin Ed= 0.15, Mmaj Ed= 97.04, Mmin Ed= 0.13, T Ed= 0.57, case name= ULS_S1, Max_Comp,

Case= 70, N Ed= 30.9, Vmaj Ed= 124.76, Vmin Ed= 0.39, Mmaj Ed= 135.22, Mmin Ed= 1.3, T Ed= 0.65, case name= ULS_S1, Max_Tens,

Case= 71, N Ed= -56.74, Vmaj Ed= -115.81, Vmin Ed= -0.52, Mmaj Ed= 192.76, Mmin Ed= -1.06, T Ed= -0.67, case name= ULS_S1, Min,

Case= 72, N Ed= -2.84, Vmaj Ed= -100.78, Vmin Ed= 0.12, Mmaj Ed= 96.19, Mmin Ed= 0.14, T Ed= 0.56, case name= ULS_S2, Max_Comp,

Case= 73, N Ed= 53.47, Vmaj Ed= 124.42, Vmin Ed= 0.52, Mmaj Ed= 136.49, Mmin Ed= 1.77, T Ed= 0.65, case name= ULS_S2, Max_Tens,

Case= 74, N Ed= -82.88, Vmaj Ed= -115.49, Vmin Ed= -0.73, Mmaj Ed= 191.49, Mmin Ed= -1.52, T Ed= -0.66, case name= ULS_S2, Min,

γ M0= 1.1

γ M1= 1.1

γ M2= 1.25

Optimize Bolt Tension+Shear combo

# Members

Primary Member: HEB340

h = 340 mm

b = 300 mm

t f= 21.5 mm

t w= 12 mm

r = 27 mm

Material: S275

f y= 265 N/mm²

f u= 410 N/mm²

thk ≤: 40 mm

b 1= 0 mm

Reinf. web pl. t  s= 0 mm

No strength check is made for members.

Secondary Member: HEA320

h = 310 mm

b = 300 mm

t f= 15.5 mm

t w= 9 mm

r = 27 mm

Material: S275

f y= 275 N/mm²

f u= 410 N/mm²

thk ≤: 16 mm

Bottom haunch:

Depth = 300 mm

Angle (≤45) = 30 °

t f haunch= 12 mm

t w haunch= 7.5 mm

α (-45°≤α≤+45°) = 0 °

# Plate & Bolts

Bolt size: M27

d = 27 mm

d 0= 30 mm

Class: 8.8

f yb= 640 N/mm²

f ub= 800 N/mm²

α v= 0.6

A = 573 mm²

A s= 459 mm²

Shear planes = 1

Conn. type: threads in shear plane

Shear cross section per bolt = 459 mm²

Plate material: S275

f yp= 265 N/mm²

f up= 410 N/mm²

thk ≤: 40 mm

Rib material: Same as primary member

# Geometry

bolt columns = 2

bolt rows 2 = 2

total bolts = 10

plate thk = 20 mm

a1 = 60 mm

a4 = 10 mm

p horiz.= 150 mm

p vert.2 = 100 mm

p vert.4 = 100 mm

a2 = 75 mm

a3 = 100 mm

depth pl.= 740 mm

width pl.= 300 mm

a vert. pl. top= 60 mm

a vert. pl. bottom= 110 mm

bolt gr. ┴ ecc. = 0 mm BoltGroupCenter distance from Conn. Axis

k wc= 1

bolt rows 4 = 2

rib thk = 15 mm

# Weld, Notch & Stiffeners

Weld between plate and sec. member:

f u,weld= 410 N/mm²

β w= 0.85

Web weld: complete penetration

l web= 525 mm

Flange weld: complete penetration

l total, flange= 900 mm

ecc. web weld= -20 mm Weld distance from Conn. Axis

Stiffeners

Continuity plate top flange

t p= 15 mm

Continuity plate bottom flange

t p= 15 mm

Continuity plate bottom haunch flange

t p= 15 mm

Diagonal stiffeners: None

Intermediate stiffeners: Col. side: Beam side:

Group 2: None None

Group 4: None None

Backing Plates:

Top: 0

Bottom: 0

# SumUp

Bolt shear: ok 17% (28)

Bolt min. distances: ok

Pl. bearing hor.: ok 1% (29)

Pl. bearing vert.: ok 10% (28)

Bolt tension: ok 38% (61)

Bolt combined: ok 45% (61)

Moment cap.: ok 59% (29)

Col. web compr.: ok 72% (29)

Beam fl.+web compr.: ok 59% (29)

Col. web tens.: ok 45% (61)

Col. web shear: ok 92% (59)

Stiffener min. thk: ok

Col. web stiff. min. thk: ok

# Bolts

Case 29

Shear:

F v,Ed,hor,bolt gr.= -27.09 kN  , Vmin,Ed

F v,Ed,vert,bolt gr.= -215 kN  , Vmaj,Edcos(α)+NEdsin(α)

M Ed,bolt gr.= -1.21 kN*m  , TEd

M ecc,Ed,bolt gr.= -1.21 kN*m  , Fv,Ed,vert,bolt gr.*bolt gr. ecc.+MEd,bolt gr.

β Lf= 0.969  =0.75≤ 1-570(mm)-15*27(mm)200*27(mm)≤1, 1-Lj-15d200d ; 0.75≤βLf≤1  (3.5)#00

F v,Rd,1 bolt= 176.3 kN  =0.6*800(N/mm²)*459(mm²)*11.25, αv fub A nsh.pl.γM2 (ta. 3.4)#00

F v,Rd,bolt gr.= 1430 kN  , Bolt eccentr. calc. method: Elastic #26

F v,Ed,bolt gr.= 216.7 kN  =-27.09(kN)2+-215(kN)2,

Bolt shear: ok 15%  , Fv,Ed,bolt gr.Fv,Rd,bolt gr.

Tension:

N Ed,bolt gr.= -264.3 kN  , NEdcos(α)-Vmaj,Edsin(α)

M ┴Ed,bolt gr.= 320.2 kN*m  =320.2(kN*m)+-215(kN)/1*0(mm),

N Rd,1 bolt= 264.4 kN  =0.9*800(N/mm²)*459(mm²)1.25, 0.9 fub AsγM2  (ta 3.4) #00

N Rd,bolt gr.= 1799 kN  , Moment calc. method: Eurocode/BS #42

N totEd,bolt gr.= 594.5 kN  , Moment calc. method: Eurocode/BS #42

Bolt tension: ok 33%  , Ntot,Ed,bolt gr.NRd,bolt gr.

Bolt combined: ok 44%  , Fv,Ed,1 boltFv,Rd,1 bolt+NEd,1 bolt1.4*NRd,1 bolt - Bolt 5

M ┴Rd,bolt gr.= 541.4 kN*m  , Moment calc. method: Eurocode/BS #42

Moment cap.: ok 59%  , M┴Ed,bolt gr.M┴Rd,bolt gr.

T-STUB DETAILS, bolt group 1

Positive Moment

Row 1: FT_Rd=377.2kN; Beam side; Individual; Mode 2; leff=292.5mm

Row 2: FT_Rd=415.3kN; Beam side; Individual; Mode 2; leff=431.1mm

Row 3: FT_Rd=310.3kN; Beam side; Group*: Rows 2 to 3: Mode 2; leff=545mm

Row 4: FT_Rd=403.8kN; Beam side; Individual; Mode 2; leff=396.3mm

Row 5: FT_Rd=292.8kN; Beam side; Group*: Rows 4 to 5: Mode 1; leff=509.6mm

Negative Moment

Row 5: FT_Rd=401.4kN; Beam side; Individual; Mode 2; leff=389mm

Row 4: FT_Rd=295.2kN; Beam side; Group*: Rows 5 to 4: Mode 1; leff=509.6mm

Row 3: FT_Rd=401.6kN; Beam side; Individual; Mode 2; leff=389.7mm

Row 2: FT_Rd=324kN; Beam side; Group*: Rows 3 to 2: Mode 2; leff=545mm

Row 1: no tension action

Details, Pos. Mom.

Row 1

Column side:

Individual: Mode1: (4*(Min(2π*47.4, Max(2*47.4+0.625*75+60, α(60.2)(=6.3)*47.4-(2*47.4+0.625*75)+60), Max(4*47.4+1.25*75, α(60.2)(=6.3)*47.4)))*0.25*21.5^2*265/1.1)/47.4= 507.8kN;

Mode2: (2*(Min(2π*47.4, Max(2*47.4+0.625*75+60, α(60.2)(=6.3)*47.4-(2*47.4+0.625*75)+60), Max(4*47.4+1.25*75, α(60.2)(=6.3)*47.4)))*0.25*21.5^2*265/1.1+59.2*264.4*2*1E3)/(47.4+59.2)= 406.6kN;

Mode3: 264.4*2= 528.8kN;

Web tension: N.A.

Beam side:

Individual: Mode1: (4*(Min(2π*67.5, Max(2*67.5+0.625*75+60, α(60)(=6.1)*67.5-(2*67.5+0.625*75)+60), Max(4*67.5+1.25*75, α(60)(=6.1)*67.5)))*0.25*20^2*265/1.1)/67.5= 417.6kN;

Mode2: (2*(Min(2π*67.5, Max(2*67.5+0.625*75+60, α(60)(=6.1)*67.5-(2*67.5+0.625*75)+60), Max(4*67.5+1.25*75, α(60)(=6.1)*67.5)))*0.25*20^2*265/1.1+75*264.4*2*1E3)/(67.5+75)= 377.2kN;

Mode3: 264.4*2= 528.8kN;

Web tension: N.A.

Triang.limit not to be applied

Row 2

Column side:

Individual: Mode1: (4*(Min(2π*47.4, Max(4*47.4+1.25*75, α(59.8)(=6.3)*47.4)))*0.25*21.5^2*265/1.1)/47.4= 699.7kN;

Mode2: (2*(Min(2π*47.4, Max(4*47.4+1.25*75, α(59.8)(=6.3)*47.4)))*0.25*21.5^2*265/1.1+59.2*264.4*2*1E3)/(47.4+59.2)= 449.2kN;

Mode3: 264.4*2= 528.8kN;

Web tension: N.A.

Beam side:

Individual: Mode1: (4*(Min(2π*70.5, Max(4*70.5+1.25*75, α(59.5)(=6.1)*70.5)))*0.25*20^2*265/1.1)/70.5= 589.2kN;

Mode2: (2*(Min(2π*70.5, Max(4*70.5+1.25*75, α(59.5)(=6.1)*70.5)))*0.25*20^2*265/1.1+75*264.4*2*1E3)/(70.5+75)= 415.3kN;

Mode3: 264.4*2= 528.8kN;

Web tension: N.A.

Triang.limit not to be applied

Row 3

Column side:

Individual: Mode1: (4*(Min(2π*47.4, Max(4*47.4+1.25*75, α(94.8)(=6.1)*47.4)))*0.25*21.5^2*265/1.1)/47.4= 677.7kN;

Mode2: (2*(Min(2π*47.4, Max(4*47.4+1.25*75, α(94.8)(=6.1)*47.4)))*0.25*21.5^2*265/1.1+59.2*264.4*2*1E3)/(47.4+59.2)= 444.3kN;

Mode3: 264.4*2= 528.8kN;

Web tension: N.A.

Group*: Rows 2 to 3: Mode1: (4*(Min(402.9, 402.9))*0.25*21.5^2*265/1.1)/47.4= 946.6kN;

Mode2: (2*(402.9)*0.25*21.5^2*265/1.1+59.2*264.4*4*1E3)/(47.4+59.2)= 797.9kN;

Mode3: 264.4*4= 1057.5kN;

Web tension: N.A.

Beam side:

Individual: Mode1: (4*(Min(2π*70.5, Max(4*70.5+1.25*75, α(119.5)(=5.5)*70.5)))*0.25*20^2*265/1.1)/70.5= 532.6kN;

Mode2: (2*(Min(2π*70.5, Max(4*70.5+1.25*75, α(119.5)(=5.5)*70.5)))*0.25*20^2*265/1.1+75*264.4*2*1E3)/(70.5+75)= 401.6kN;

Mode3: 264.4*2= 528.8kN;

Web tension: N.A.

Group*: Rows 2 to 3: Mode1: (4*(Min(545, 545))*0.25*20^2*265/1.1)/70.5= 744.9kN;

Mode2: (2*(545)*0.25*20^2*265/1.1+75*264.4*4*1E3)/(70.5+75)= 725.6kN;

Mode3: 264.4*4= 1057.5kN;

Web tension: N.A.

Triang.limit not to be applied

Row 4

Column side:

Individual: Mode1: (4*(Min(2π*47.4, Max(4*47.4+1.25*75, α(100)(=6.1)*47.4)))*0.25*21.5^2*265/1.1)/47.4= 675.5kN;

Mode2: (2*(Min(2π*47.4, Max(4*47.4+1.25*75, α(100)(=6.1)*47.4)))*0.25*21.5^2*265/1.1+59.2*264.4*2*1E3)/(47.4+59.2)= 443.9kN;

Mode3: 264.4*2= 528.8kN;

Web tension: N.A.

Beam side:

Individual: Mode1: (4*(Min(2π*70.5, Max(4*70.5+1.25*75, α(100)(=5.6)*70.5)))*0.25*20^2*265/1.1)/70.5= 541.7kN;

Mode2: (2*(Min(2π*70.5, Max(4*70.5+1.25*75, α(100)(=5.6)*70.5)))*0.25*20^2*265/1.1+75*264.4*2*1E3)/(70.5+75)= 403.8kN;

Mode3: 264.4*2= 528.8kN;

Web tension: N.A.

Triang.limit not to be applied

Row 5

Column side:

Individual: Mode1: (4*(Min(2π*47.4, Max(4*47.4+1.25*75, α(86.5)(=6.1)*47.4)))*0.25*21.5^2*265/1.1)/47.4= 682.2kN;

Mode2: (2*(Min(2π*47.4, Max(4*47.4+1.25*75, α(86.5)(=6.1)*47.4)))*0.25*21.5^2*265/1.1+59.2*264.4*2*1E3)/(47.4+59.2)= 445.4kN;

Mode3: 264.4*2= 528.8kN;

Web tension: N.A.

Group*: Rows 4 to 5: Mode1: (4*(Min(394.6, 394.6))*0.25*21.5^2*265/1.1)/47.4= 927kN;

Mode2: (2*(394.6)*0.25*21.5^2*265/1.1+59.2*264.4*4*1E3)/(47.4+59.2)= 793.5kN;

Mode3: 264.4*4= 1057.5kN;

Web tension: N.A.

Beam side:

Individual: Mode1: (4*(Min(2π*70.5, Max(4*70.5+1.25*75, (4*70.5+1.25*75+α(88)(=5.7)*70.5)/2)))*0.25*20^2*265/1.1)/70.5= 531.8kN;

Mode2: (2*(Min(2π*70.5, Max(4*70.5+1.25*75, (4*70.5+1.25*75+α(88)(=5.7)*70.5)/2)))*0.25*20^2*265/1.1+75*264.4*2*1E3)/(70.5+75)= 401.4kN;

Mode3: 264.4*2= 528.8kN;

Web tension: N.A.

Group*: Rows 4 to 5: Mode1: (4*(Min(509.6, 509.6))*0.25*20^2*265/1.1)/70.5= 696.6kN;

Mode2: (2*(509.6)*0.25*20^2*265/1.1+75*264.4*4*1E3)/(70.5+75)= 713.9kN;

Mode3: 264.4*4= 1057.5kN;

Web tension: N.A.

Triang.limit not to be applied

Details, Neg. Mom.

Row 5

Column side:

Individual: Mode1: (4*(Min(2π*47.4, Max(4*47.4+1.25*75, α(86.5)(=6.1)*47.4)))*0.25*21.5^2*265/1.1)/47.4= 682.2kN;

Mode2: (2*(Min(2π*47.4, Max(4*47.4+1.25*75, α(86.5)(=6.1)*47.4)))*0.25*21.5^2*265/1.1+59.2*264.4*2*1E3)/(47.4+59.2)= 445.4kN;

Mode3: 264.4*2= 528.8kN;

Web tension: N.A.

Beam side:

Individual: Mode1: (4*(Min(2π*70.5, Max(4*70.5+1.25*75, (4*70.5+1.25*75+α(88)(=5.7)*70.5)/2)))*0.25*20^2*265/1.1)/70.5= 531.8kN;

Mode2: (2*(Min(2π*70.5, Max(4*70.5+1.25*75, (4*70.5+1.25*75+α(88)(=5.7)*70.5)/2)))*0.25*20^2*265/1.1+75*264.4*2*1E3)/(70.5+75)= 401.4kN;

Mode3: 264.4*2= 528.8kN;

Web tension: N.A.

Triang.limit not to be applied

Row 4

Column side:

Individual: Mode1: (4*(Min(2π*47.4, Max(4*47.4+1.25*75, α(100)(=6.1)*47.4)))*0.25*21.5^2*265/1.1)/47.4= 675.5kN;

Mode2: (2*(Min(2π*47.4, Max(4*47.4+1.25*75, α(100)(=6.1)*47.4)))*0.25*21.5^2*265/1.1+59.2*264.4*2*1E3)/(47.4+59.2)= 443.9kN;

Mode3: 264.4*2= 528.8kN;

Web tension: N.A.

Group*: Rows 5 to 4: Mode1: (4*(Min(394.6, 394.6))*0.25*21.5^2*265/1.1)/47.4= 927kN;

Mode2: (2*(394.6)*0.25*21.5^2*265/1.1+59.2*264.4*4*1E3)/(47.4+59.2)= 793.5kN;

Mode3: 264.4*4= 1057.5kN;

Web tension: N.A.

Beam side:

Individual: Mode1: (4*(Min(2π*70.5, Max(4*70.5+1.25*75, α(100)(=5.6)*70.5)))*0.25*20^2*265/1.1)/70.5= 541.7kN;

Mode2: (2*(Min(2π*70.5, Max(4*70.5+1.25*75, α(100)(=5.6)*70.5)))*0.25*20^2*265/1.1+75*264.4*2*1E3)/(70.5+75)= 403.8kN;

Mode3: 264.4*2= 528.8kN;

Web tension: N.A.

Group*: Rows 5 to 4: Mode1: (4*(Min(509.6, 509.6))*0.25*20^2*265/1.1)/70.5= 696.6kN;

Mode2: (2*(509.6)*0.25*20^2*265/1.1+75*264.4*4*1E3)/(70.5+75)= 713.9kN;

Mode3: 264.4*4= 1057.5kN;

Web tension: N.A.

Triang.limit not to be applied

Row 3

Column side:

Individual: Mode1: (4*(Min(2π*47.4, Max(4*47.4+1.25*75, α(94.8)(=6.1)*47.4)))*0.25*21.5^2*265/1.1)/47.4= 677.7kN;

Mode2: (2*(Min(2π*47.4, Max(4*47.4+1.25*75, α(94.8)(=6.1)*47.4)))*0.25*21.5^2*265/1.1+59.2*264.4*2*1E3)/(47.4+59.2)= 444.3kN;

Mode3: 264.4*2= 528.8kN;

Web tension: N.A.

Beam side:

Individual: Mode1: (4*(Min(2π*70.5, Max(4*70.5+1.25*75, α(119.5)(=5.5)*70.5)))*0.25*20^2*265/1.1)/70.5= 532.6kN;

Mode2: (2*(Min(2π*70.5, Max(4*70.5+1.25*75, α(119.5)(=5.5)*70.5)))*0.25*20^2*265/1.1+75*264.4*2*1E3)/(70.5+75)= 401.6kN;

Mode3: 264.4*2= 528.8kN;

Web tension: N.A.

Triang.limit not to be applied

Row 2

Column side:

Individual: Mode1: (4*(Min(2π*47.4, Max(4*47.4+1.25*75, α(59.8)(=6.3)*47.4)))*0.25*21.5^2*265/1.1)/47.4= 699.7kN;

Mode2: (2*(Min(2π*47.4, Max(4*47.4+1.25*75, α(59.8)(=6.3)*47.4)))*0.25*21.5^2*265/1.1+59.2*264.4*2*1E3)/(47.4+59.2)= 449.2kN;

Mode3: 264.4*2= 528.8kN;

Web tension: N.A.

Group*: Rows 3 to 2: Mode1: (4*(Min(402.9, 402.9))*0.25*21.5^2*265/1.1)/47.4= 946.6kN;

Mode2: (2*(402.9)*0.25*21.5^2*265/1.1+59.2*264.4*4*1E3)/(47.4+59.2)= 797.9kN;

Mode3: 264.4*4= 1057.5kN;

Web tension: N.A.

Beam side:

Individual: Mode1: (4*(Min(2π*70.5, Max(4*70.5+1.25*75, α(59.5)(=6.1)*70.5)))*0.25*20^2*265/1.1)/70.5= 589.2kN;

Mode2: (2*(Min(2π*70.5, Max(4*70.5+1.25*75, α(59.5)(=6.1)*70.5)))*0.25*20^2*265/1.1+75*264.4*2*1E3)/(70.5+75)= 415.3kN;

Mode3: 264.4*2= 528.8kN;

Web tension: N.A.

Group*: Rows 3 to 2: Mode1: (4*(Min(545, 545))*0.25*20^2*265/1.1)/70.5= 744.9kN;

Mode2: (2*(545)*0.25*20^2*265/1.1+75*264.4*4*1E3)/(70.5+75)= 725.6kN;

Mode3: 264.4*4= 1057.5kN;

Web tension: N.A.

Triang.limit not to be applied

*: Group values are from total values, that is to find final value upper (or lower) row values should be detracted

# Additional checks for moment connections

Case: 59

Column web in compression:

ω = 0.822  , 11+1.3beff,c,wctwc/Awc2 [β=1] (table 6.3 EC)

b eff,c,wc= 284.5 mm  =42(mm)+242.5(mm), tfb+22 ap+5tfc+rc+sp  (6.11)#00

λ p= 0.725  , 0.932 beff,c,wc dwc fy,wcE twc2 6.13c#00

ρ = 0.998  , λ'p-0,2λ'p2 6.13b #00

k wc= 1  , 1σcom,Ed ≤ 0.7 fy,wc  6.14#00

F c,wc stiff,Rd= 877.5 kN  =2*15(mm)*275(N/mm²)min144(mm)-27(mm)1.1;144(mm)*11.1, 2twc,stiff fy minbstiff-rγM0;bstiffργM1 2.112.12 #42

F c,wc,Rd= 674.6 kN  =0.822*1*284.5(mm)*12(mm)*265(N/mm²)*min11.1;0.9981.1, ωkwcbeff,c,wctwcfy,wc▪min1γM0;ργM1 6.9 #00

F c,wc,tot,Rd= 1097 kN  =Min(1097(kN),2*15(mm)*144(mm)-27(mm))*275(N/mm²)/(0.8*1.1),  minFc,wc,stiff,Rd+Fc,wc,tot,Rd;2twc,stiff fy bstiff-r0.8 γM0 2.13 #42

F c,Ed= 720.9 kN  =-331.6(kN*m)487.2(mm)+min(0,-40.18(kN)),

Col. web compr.: ok 66%  720.9(kN)1097(kN), Fc,EdFc,wc,tot,Rd

Beam flange and web in compression:

F c,fb,Rd= 1329 kN  =792.4(kN*m)610(mm)-15.5(mm)/2-12(mm)/2, Mc,Rdh-tfb 6.21 #00

Beam fl.+web compr.: ok 54%  =720.9(kN)1329(kN), Fc,EdFc,fb,Rd

Column web panel shear:

V wp,Rd= 702.1 kN  =0.9*265(N/mm²)*5609(mm²)3*1.1, 0,9Fy,wc Avc3 γM0 (6.7)#00

V wp,add,Rd= 41.52 kN  =min(4*8.352(kN*m)596.3(mm);2*8.352(kN*m)+2*4.025(kN*m)596.3(mm)), min(4Mpl,fc,Rdds;2Mpl,fc,Rd+2Mpl,st,Rdds) (6.8)#00

V wp,diag.st,Rd= 0 kN  =0(mm²)+0(mm²), steps 6E and 6A in #42

V wp,tot,Rd= 743.6 kN  =702.1(kN)+41.52(kN)+0(kN), Vwp,Rd+Vwp,add,Rd+Vwp,diag.st,Rd

V wp,Ed= 680.7 kN  =-331.6(kN*m)487.2(mm),

Col. web shear: ok 92%  =680.7(kN)743.6(kN), Vwp,EdVwp,tot,Rd

Stiffener min. thk: ok

Web tens. min. thk = 0 mm  ,

Stiffener min. thk: = 6.701 mm  , Column; between rows 1-2

Col. web stiff. min. thk: ok

Shear min. thk = 12 mm  =12(mm), tw

Column web in tension:

F t,wc stiff,Rd= 877.5 kN  =2*15(mm)*275(N/mm²)*(144(mm)-27(mm))/1.1, 2twc,stiff fy(bstiff-r)γM0

F t,wc,Rd= 694.9 kN  =0.81*296.8(mm)*12(mm)*265(N/mm²)1.1, ω beff,t,wc twc fy,wcγM0 6.15 #00

F t,wc,tot,Rd= 1572 kN  =694.9(kN)+877.5(kN), Ft,wc,stiff,Rd+Ft,wc,Rd

F t,wc,Ed= 680.7 kN  =331.6(kN*m)487.2(mm)+max{0,-40.18(kN)

Col. web tens.: ok 43%  680.7(kN)1572(kN), Ft,EdFt,wc,tot,Rd

# Plate bearing

Case: 28

Tot. ref. thk = 20 mm

p 1,min.= 66 mm

e 1,min.= 36 mm

p 2,min.= 72 mm

e 2,min.= 36 mm

p 1,opt.= 105 mm

e 1,opt.= 90 mm

p 2,opt.= 90 mm

e 2,opt.= 45 mm

Horizontal forces

p 1,hpl= 150 mm ok

e 1,hpl= 75 mm ok

p 2,hpl= 100 mm ok

e 2,hpl= 60 mm ok

α b,hpl= 0.833

α d,hpl= 0.833

k 1,hpl= 2.5

Vertical forces

p 1,vpl= 100 mm ok

e 1,vpl= 60 mm ok

p 2,vpl= 150 mm ok

e 2,vpl= 75 mm ok

α b,vpl= 0.667

α d,vpl= 0.667

k 1,pl= 2.5

F b,Rd,h,pl= 369 kN  =2.083*1*410(N/mm²)*27(mm)*20(mm)/1.25, k1,hpl αb,hpl kred,hor. fu d tγM2 (ta. 3.4)#00

F b,Ed,h,pl= 3.656 kN Bolt 10

F b,Rd,v,pl= 295.2 kN  =1.667*1*410(N/mm²)*27(mm)*20(mm)/1.25, k1,vpl αb,vpl kred,vert. fu d tγM2 (ta. 3.4)#00

F b,Ed,v,pl= 28.66 kN Bolt 10

Plate bear. h.: ok 1%

Plate bear. v.: ok 10%

# Welding

Case: 1

Web weld

No check required for complete pen. weld

Flange weld

No check required for complete pen. weld

# Stiffness

Case: 1

k 1= 4.375 mm  from ta. 6.11 #00

k 2= ∞ mm  from ta. 6.11 #00

k eq= 8.842 mm  Σkeff,r hrzeq  (6.29)#00

z eq= 487.2 mm  Σkeff,r hr2Σkeff,r hr  (6.31)#00

S j,ini,Mpos= 1.459E+05 kN*m  E zeq21 Σ1ki  (6.27)#00

S j,ini,Mneg= 9.828E+04 kN*m  E zeq21 Σ1ki  (6.27)#00

μ = 1  1 or 1.5Mj,Ed / Mj,Rdψ  (6.28)#00

S j= 1.459E+05 kN*m  E zeq2μ Σ1ki  (6.27)#00

Classification of joint by stiffness according to #00:

k br= 25 (and Kb/Kc>=0.1)

L b= 7300 mm

The connection for positive moment can be classified as semi-rigid

The connection for negative moment can be classified as semi-rigid  S j,Pin= 1.238E+04 kN*m  =0.5*2.1*105(N/mm²)*8.607*108(mm4)/7300(mm), 0.5 E Ib/Lb  (fig.5.4)#00  S j,Rigid=

6.19E+05 kN*m  =25*2.1*105(N/mm²)*8.607*108(mm4)/7300(mm), kbrE Ib/Lb  (fig.5.4)#00

# Comments/Notes

Haunch flange and web thickness should not be less than member thicknesses

Manual update turned on, click Update Calcs to see results

Env. output from joints 1138, 1138, 1139, 1139, 1140, 1140, 1141, 1141, 1142, 1142, 1143, 1143, 1144, 1144, 1149, 1150, 1151, 1151, 1152, 1152, 1154, 1154, 1155, 1155 sec. members 64, 154, 65, 153, 74, 575, 78, 151, 150, 506, 149, 930, 148, 931, 148, 149, 150, 548, 151, 160, 153, 158, 154, 157 file MODEL_V2.0.sdb.

References

#00: EN 1993-1-8: 2005, Design of steel structures, Design of joints, Cen

#26: AISC Steel Construction Manual

#42: Joints in steel constructions, Moment Connections, SCI and BCSA, 1995